Influence of boundary conditions on electronic and transport properties in monolayer low – buckled HgTe nanoribbons

D. B. Topalović1, V. V. Arsoski2, M. Ž. Tadić2, M. Radenković1, P. Božović1, and F. M. Peeters3

1Vinča Institute of Nuclear Sciences, University of Belgrade,

P. O. Box 522, 11001 Belgrade, Serbia

2School of Electrical Engineering, University of Belgrade,

P.O. Box 35 – 54, 11120 Belgrade, Serbia

3Department of Physics, University of Antwerp,

Groenenborgerlaan 171, B – 2020 Antwerp, Belgium

e-mail:dusan.topalovic@vin.bg.ac.rs

HgTe is a II-VI compound semi-metal with inverted band ordering, low effective mass and high electron mobility [1]. Nanostructures based on this compound are intensively studied due to their specific topological properties [2]. In HgTe based nanostructures an inversion in the band ordering is accompanied by a quantum phase transition from a normal insulator to the phase of a nontrivial two-dimensional (2D) topological insulator or the Quantum Spin Hall insulator [3]. A good dynamic stability of HgTe monolayer is predicted from first-principles methods [4], which opened the possibility to use this material in thin-layer electronic devices. Even before these theoretical findings, HgTe nanoribbons have been experimentally realized [5], and a topological field effect quantum transistor was proposed [6].

We investigate the electronic and transport properties of low-buckled 2D HgTe zig-zag and armchair nanoribbons. Modeling of electronic states was performed by using a single-particle tight-binding model in the nearest-neighbour approximation [7]. The sp3d 5s* basis set in the Slater-Koster notation was used. The spin-orbit interaction was included in the model by using the Chadi formalism [8].

We found specific edge states in the range of energies that define the fundamental band gap of 2D HgTe monolayer. Moreover, we show that the electronic and transport properties of these states depend strongly on nanoribbon width and type of edge, and we demonstrate how external fields can be employed to control them.

REFERENCES

[1] E.O. Melezhik, J. V. Gumenjuk – Sichevska, and F. F. Sizov, SpringerPlus. 5, 80 (2016).

[2] D. B. Topalović, V. V. Arsoski, M. Ž. Tadić, and F. M. Peeters, Phys. Rev. B 100, 125304 (2019).

[3] B. A. Bernevig, T. L. Hughes, S. C. Zhang, Science 314, 1757 (2006).

[4] H. Zheng, X. B. Li, N. K. Chen, S. Y. Xie, W. Q. Tian, Y. Chen, H. Xia, S. B. Zhang,  and H. B. Sun, Phys. Rev. B 92, 115307 (2015).

[5] F. S. Sangsefidi, N. Mir, and M. Salavati-Niasari, Mat. Sci. Semicon. Proc. 27, 500 (2014).

[6] H. H. Fu, J. H. Gao,  and K. L. Yao, Nanotechnology 25, 225201 (2014).

[7] G. Allan and C. Delerue, Phys. Rev. B 86, 165437 (2012).

[8] D. J. Chadi, Phys. Rev. B 16, 790 (1977).