Raman spectroscopy and multivariate classification as a tool for different ketchup samples discrimination

S. Kolasinac1, I. Pecinar1, Z. Dajic-Stevanovic1

1 Faculty of Agriculture, Departman of Agricultural Botany, Belgrade, Serbia

e-mail:stefan.kolasinac@agrif.bg.ac.rs

The main aim of this paper is to test PCA-LDA (Principal Component Analysis – Linear Discriminant Analysis) coupled with Raman spectroscopy for discrimination of two ketchups commercially available at the local markets in Serbia. Raman spectra were recorded at two different wavelengths (785 and 532 nm) and 30 spectra per sample are obtained. The data were divided into the training (3/4 of samples) and validation (1/4 of samples) data. Two types of pre-processing methods which were applied and results of discrimination are represented in table 1. Obtained results showed that second-order derivative did not improve discrimination power. On the other hand, laser at 785 nm provided a better classification of samples which can be related to the fact that 785 nm laser reveals bands that are masked by the high fluorescence background seen when using the 532 nm laser and consequently gives wider chemical information about the sample (Haraa et al., 2018). Pre-processing analysis of the spectra was performed using the software The Unscrambler X version 10.4 (Camo Software, Oslo, Norway) while supervised classification models were performed using the Python software.

Table 1. Classification results of training and test sets of PCA-LDA

| Excitation wavelength |  |  | 785 nm |  | 532 nm |  |
| --- | --- | --- | --- | --- | --- | --- |
| Pre-processing methods |  |  | Smoothing + baseline correction + normalization + principal component analysis | Smoothing + baseline correction + normalization + 2nd order derivative + principal component analysis | Smoothing + baseline correction + normalization + principal component analysis | Smoothing + baseline correction + normalization + 2nd order derivative + principal component analysis |
| Correct classified (%) | Training data | Sample 1 | 95.45 | 95.45 | 95.45 | 90.91 |
|  |  | Sample 2 | 100.00 | 95.45 | 95.45 | 95.45 |
|  | Test data | Sample 1 | 87.50 | 87.50 | 62.50 | 37.50 |
|  |  | Sample 2 | 100.00 | 87.50 | 37.50 | 37.50 |

REFERENCES

[1] R. Haraa, M. Ishigakib, Y.  Kitahamab, Y.   Ozakib, T.     Genkawaa. Food Chem. 258, 308 (2018)