Deep learning solutions for cross-phase modulation dominated channels

Lazar S. Stojković1, Velimir M. Ilić2 and Ivan B. Djordjevic3

1Faculty of Science and Mathematics, University of Niš, Serbia

e-mail: lazar.stojkovic@pmf.edu.rs

2Faculty of Science and Mathematics, University of Niš, Serbia

e-mail: velimir.ilic@pmf.edu.rs

3Department of Electrical and Computer Engineering,

College of Engineering, University of Arizona, USA

e-mail: ivan@email.arizona.edu

Optical fiber, unlike other transmission media, features significant change of light propagation properties with increasing signal power, known as Kerr-nonlinear effect, which induces self-phase modulation (SPM) of the transmitted signal [1, 5]. Moreover, the wavelength of light in one wavelength channel can affect the phase of the wavelength of light in nearby channels, inducing a nonlinear optical effect known as cross-phase modulation (XPM) [2]. We hereby present the artificial neural networks (ANN) as an efficient solution for symbol detection and constellation design problems for the XPM-dominated systems as in [2].

In case of symbol detection, some already proposed constellations have been considered for a wide range of nonlinearity intensities, and our ANN detector has been compared to some established detectors such as minimum-distance and two-stage detectors [2, 3], where it exhibits performance superiority while preserving low complexity. In addition, we advance auto-encoder technique previously used for SPM dominated channels [4] and adapt it for the XPM case, which allows precise learning of constellations for specific fiber channel settings and power constraints, in significant improvements in symbol error rates (SER).

REFERENCES

[1] R. J. Essiambre, G. Kramer, P. J. Winzer, G. J. Foschini, B. Goebel, “Capacity Limits of Optical Fiber Networks”, Journal of Lightwave Technology, vol. 28, no. 4, pp. 662-701, Feb.15, 2010, doi:10.1109/JLT.2009.2039464.

[2] T. Liu, I. Djordjevic, M. Li, “Signal constellation design for cross-phase modulation dominated channels”, ICTON 2015 - 17th International Conference on Transparent Optical Networks, 2015, doi:1-4. 10.1109/ICTON. 2015.7193299.

[3] L. Beygi, E. Agrell, M. Karlsson, “Optimization of 16-point Ring Constellations in the Presence of Nonlinear Phase Noise”, Optical Fiber Communication Conference and Exposition (OFC) and The National Fiber Optic Engineers Conference (NFOEC), 2011, doi:10.1364/OFC.2011.OThO4.

[4] S. Li, C. Häger, N. Garcia and H. Wymeersch, “Achievable Information Rates for Nonlinear Fiber Communication via End-to-end Autoencoder Learning”, 2018 European Conference on Optical Communication (ECOC), 2018, pp. 1-3, doi:10.1109/ECOC.2018.8535456.

[5] L. Mo, N. Stojanovic, V. Ilic, I. Djordjevic, F. Kuppers, “Target constellation diagram determination method, data sending method, and aparatus", EP3490207A4 European Patent Office, Jul. 24, 2019.