Rosette based metamaterial for circularly polarized terahertz waves manipulation

D. B. Stojanovic1, U. Ralevic2, Y. Demirhan 3, G. Aygun3 L. Ozyuzer3,4

1Vinca Institute of Nuclear Sciences-National Institute of Republic of Serbia, University of Belgrade

Belgrade, Serbia

2Institute of physics, University of Belgrade, Belgrade, Serbia

3 Department of Physics, Izmir Institute of Technology, Izmir, Turkey

4 Teknoma Technological Materials Industrial and Trading Inc., Izmir, Turkey

e-mail:dankas@vin.bg.ac.rs

Metamaterials that respond resonantly in interaction with the terahertz (THz) electromagnetic waves are significant for the achievement of diverse optical functionalities in THz spectral range [1]. Also, they are virtually desirable platforms for investigating chiral effects which rise due to different interaction of metamaterials with left and right circularly polarized light [2, 3].  Light polarization is an important feature of electromagnetic waves and manipulation of polarization plays pivotal role in various areas such as communications, imaging and sensing. One of potential applications of the THz metadevices is for protein quality control in the biotechnology or food industries [4].

Chiral properties of metamaterial can be modified via different external influences, such as by changing the interlayer twist angle, adding a dielectric spacer, or modulating its thickness. Twisting or rotating achiral layered structures in parallel planes enables engineering of the extrinsic chirality and consequential optical performance. For example, the twist angle from counterclockwise rotation of an upper layer with respect to the one under it, leads to a left-handed stacking geometry [5]. In this study, we performed numerical simulations of the metamaterial structure in terahertz frequency range, 0.25-0.75 THz. The numerical simulations were done for the case of two parallel gold rosettes on sapphire substrates in which we analyzed the influence of rotating one rosette with respect to the another on chiral properties. Two geometries of the rosette based structures were studied. First geometry is based on resonator which consists of two same parallel rosettes and the second one of two parallel rosettes, but one of them is trimmed.

Acknowledgment: This research is partially supported by TUBITAK (Scientific and Technical Research Council of Turkey) project number 220N350 which is a bilateral project with Vinča Institute of Nuclear Sciences, Belgrade, Serbia which is supported by Ministry of Science, Technological Development and Innovations of the Republic of Serbia (Grant 451-03-68/2022-14/ 200017).

REFERENCES

[1] M. Tonouchi, Nat. Photonics 1, 97 (2007).

[2] D. B. Stojanović, P. P. Beličev, G. Gligorić, Lj. Hadžievski, J. of Phys. D: Appl. Phys. 51 (4), 045106 (2018).

[3] Z. Wang, F. Cheng, T. Winsor, Y. Liu, Nanotechnology 27, 412001 (2016).

[4] Z. Zhang et al, Biomed. Opt. Express 13(1), 209 (2022).

[5] Z. Han et al., Adv. Mater. 35, 2206141 (2023).