Optical interconnects and filters based on waveguide arrays

Jelena Krsic1, Mirjana Stojanovic1, Kolja Bugarski1,2, Nikola Stojanovic3, Aleksandra Maluckov1,

Peter Veerman4, Jovana Petrovic1

1Vinca Institute of Nuclear Sciences, National Institute of the Republic of Serbia, University of Belgrade, Serbia

2School of Electrical Engineering, University of Belgrade, Serbia

3Deutsches Zentrum für Luft- und Raumfahrt (DLR),Berlin, Germany

4Portland State University, Oregon, US

E-mail: mirjana.stojanovic@vin.bg.ac.rs

Rapidly increasing demand for higher data bandwidths has motivated exploration of new communication channels based on spatially multiplexed in-fibre and on-chip coupled light guides [1]. However, the conventionally used periodically arranged coupled waveguides display complicated light propagation patterns, ranging from quasiperiodic to nearly chaotic. Taking a different approach, we spectrally engineer interwaveguide coupling to instigate self-imaging of the input light state at the array output and thus enable construction of novel high-fidelity interconnects [2]. Simple implementation via modulation of the interwaveguide separations makes these interconnects realizable in all fabrication platforms.

Moreover, the wavelength dependent self-imaging opens up possibilities for construction of new multiplexing devices [3].  Here, we present designs of band-pass filters and dichroic splitters for VIS and NIR and propose the strategies for selection of their central wavelengths and bandwidths.

Figure 1. a) Self-imaging in a SiN array composed of 5 waveguides. b) Transmission and fidelity of transfer after the 1st revival period at z = 0.36 mm [3].

REFERENCES

[1] B.J. Puttnam, G. Rademacher, R.S. Luís, Optica 8, 1186 (2021).

[2] J. Petrovic, J. Veerman, Ann. Phys. 392, 128 (2018).

[3] J. Petrovic, J. Krsic et al., J. Opt. Las. Technol. 163, 109381 (2023).