Novel Phosphorene-based Photodetector 
for biomarker analysis at UV range

Michał Suplewski1, Małgorzata Szczerska1 , Paweł Jakóbczyk1

1Department of Metrology and Optoelectronics, Faculty of Electronics, Telecommunications and Informatics, Gdańsk University of Technology, Poland

e-mail: suplewskimichal@gmail.com

We present a novel ultraviolet (UV) photodetector based on few-layer black phosphorus (FLBP) encapsulated in nitrocellulose, enabling highly stable and sensitive detection of UV light within the solar-blind spectral region, specifically around 260-280 nm. The device exhibits a pronounced and reversible change in electrical resistance upon exposure to 260-280 nm UV radiation, demonstrating its applicability for bio-photonic sensing without the need for external optical filters. Encapsulation in nitrocellulose significantly enhances environmental stability, overcoming the rapid degradation issues typical of black phosphorus materials [1-5].

The capability of this detector to sensitively measure UV absorption near 270 nm opens new possibilities for biomolecular analysis, particularly in the quantification and purity assessment of nucleic acids such as DNA and RNA. DNA and RNA nucleotides exhibit strong absorption maxima near 270 nm due to their aromatic bases; precise measurement in this spectral range enables rapid and non-destructive quantification of nucleic acid concentration and purity, essential in molecular biology and biomedical diagnostics [6].

The use of UV absorbance ratios (e.g., A260/A280) is a widely established method to assess sample contamination by proteins or other impurities [6-7]. Our FLBP-based UV detector provides a compact, flexible, and highly sensitive platform for such measurements, potentially integrating into portable biomedical devices for real-time nucleic acid quality monitoring.

REFERENCES

[1] M. Szczerska et al.,  “Investigation of the Few-Layer Black Phosphorus Degradation by the Photonic Measurements” Adv. Mater. Interfaces 10, 2202289 (2023)

[2] P. Jakóbczyk et al.,  “Low-power microwave-induced fabrication of functionalised few-layer black phosphorus electrodes: A novel route towards Haemophilus Influenzae pathogen biosensing devices” Applied Surface Science, 539, 148286 (2021)

[3] D. Roy et al., "Advancement in phosphorene: Synthesis, properties, and applications," Applied Materials Today, 35, 101944 (2023).

[4] W. Li et al., "Phosphorene Degradation: Visualization and Quantification of Nanoscale Phase Evolution by Scanning Transmission X-ray Microscopy," Chem. Mater., 32, 1272–1280 (2020).

[5] L. Peng et al., "Black Phosphorus: Degradation Mechanism, Passivation Method, and Application for In Situ Tissue Regeneration," Adv. Materials Interfaces, 7, 2001538 (2020).

[6] Thermo Fisher Scientific, "Extinction Coefficients and Purity of Nucleic Acids," Application Note TR0006, https://assets.thermofisher.com/TFS-Assets/LSG/Application-Notes/TR0006-Extinction-coefficients.pdf.

[7] Technology Networks, "A guide to DNA and RNA quantification and quality," https://www.technologynetworks.com/genomics/articles/a-guide-to-dna-and-rna-quantification-and-quality-392900.

ACKNOWLEGMENT: This project is supported by  DS programs of the Faculty of Electronics, Telecommunications and Informatics of Gdańsk Tech, 3/1/2024/IDUB/III.1a/Ra grant under the Radium Learning Through Research program at Gdańsk Tech, 7/1/2024/IDUB/III.4c/Tc grant under the Technetium Talent Management Grant, DEC-6/1/2025/IDUB/III.4a/Pu, 2025 under the Plutonium Supporting Student Research Teams program at Gdańsk Tech, Ministry of Science and Higher Education of the Republic of Poland under agreement no. MNiSW/2024/DPI/433. Additional support was provided by the COST Action CA21159.