Single Shot Two-Dimensional Polarization Mapping of Birefringent Elements and Devices

M. Mincheva1, A. Stefanov2,3, I. Stefanov1,4, L. Stoyanov1,4, A. Dreischuh1,4,5

1Department of Quantum Electronics, Faculty of Physics,

Sofia University "St. Kliment Ohridski", Sofia, Bulgaria

2 Department of Mechatronics, Robotics and Mechanics, Faculty of Mathematics and Informatics,

Sofia University “St. Kliment Ohridski”, Sofia, Bulgaria

3 Institute of Mathematics and Informatics, Bulgarian Academy of Sciences, Sofia, Bulgaria

4National Centre of Excellence Mechatronics and Clean Technologies, 8 bul. Kliment Ohridski, Sofia, Bulgaria

5Bulgarian Academy of Sciences, Sofia, Bulgaria

e-mail: mariatm@uni-sofia.bg

Light beam passing through different anisotropic elements undergoes a change in its polarization state. If one wants to analyze a more complex polarization structuring (e.g. radial, azimuthal or more sophisticated [1-3]), a suitable method for measuring the polarization in two transverse dimensions could be exploited. An intuitive example of where one can use the polarization mapping is the structured light in the form of polarization vortices. Here we present one such mapping method denoted as two-dimensional polarization mapping.

Conventionally, one records transmitted intensities through combinations of rotating polarizers, wave plates and imaging detectors under different analyzer settings. From these measurements full Stokes parameter maps or local polarization ellipses are reconstructed across the beam profile.

In this work, we implemented an innovative single-shot polarization mapping technique for birefringent devices, which employs a 9×9 array of probe sub-beams to capture spatially resolved polarization information in a single measurement. This means that the two-dimensional polarization distribution is reconstructed in only one step of data acquisition. The measurement time is reduced while the high spatial resolution remains preserved. We first performed a test measurement with commercially-available polarizing vortex plates converting the linearly polarized light to radially/azimuthally polarized light. To demonstrate the robustness of the technique, we applied the same approach to quantify the polarization transfer function of reflective phase-only liquid crystal on silicon spatial light modulator. The results show that the reported method is particularly useful for fast polarization monitoring and control in advanced photonics applications.

We acknowledge funding of the Bulgarian National Science Fund (project KΠ-06-H78/6). The work was also supported by the Bulgarian Ministry of Education and Science as a part of National Roadmap for Research Infrastructure, project ELI ERIC BG. M.M., L.S. and A.D. were also supported by the European Union NextGenerationEU through the “National Recovery and Resilience Plan of the Republic of Bulgaria, project BG-RRP-2.004-0008-C01”. Research equipment of the project № BG16RFPR002-1.014-0006 "National Centre of Excellence Mechatronics and Clean Technologies" was used for experimental work financially supported by European Regional Development Fund under "Research Innovation and Digitization for Smart Transformation" program 2021-2027.

REFERENCES

[1] X.Wang, F. Yang, J. Yin, “Mapping the polarization distribution of arbitrary vector polarization beam”, Optik, 144, 124-131, (2017).

[2] B. Schaefer, E. Collett, R. Smyth, D. Barrett, B. Fraher, “Measuring the Stokes polarization parameters” Am. J. Phys.75 (2), 163–168 (2007)

[3] J. Yang, D. Lin, D. Bao, S.Tao, “Pixel level control of amplitude, phase, and polarization of an arbitrary vector beam” Appl. Phys. Lett., 121 (19), (2022).