Multimodal optical imaging and label-free sensing on-chip for monitoring intracellular and extracellular cellular processes

F. Torrini1,2, M. Rojas Rodríguez2, C. Dallari1,2, L. Ponticelli1,2, A. Jakovljevic3, M. Catarzi2, T. Pajic3, S. Donato1,4, A. Jakovljevic3, C. Capitini2, M. Calamai2, P. Andjus3, B. Jelenkovic5, S. Scarano6, S. Nocentini1,4, C. Credi1,2

1National Institute of Optics, National Research Council (INO-CNR), Via Nello Carrara 1, 50019 Sesto Fiorentino, Italy

2European Laboratory for Non-linear Spectroscopy (LENS), Via Nello Carrara 1, 50019 Sesto Fiorentino, Italy

3Institute of Physiology and Biochemistry “Ivan Djaja”, Faculty of Biology, University of Belgrade, Studentski trg 3, 11001, Belgrade, Serbia

4National Institute of Metrology (INRiM), Strada delle Cacce 91, 10135, Turin, Italy

5Institute of Physics Belgrade, University of Belgrade, National Institute of the Republic of Serbia, Pregrevica 118, 11080 Belgrade, Serbia

6Department of Chemistry “Ugo Schiff’, University of Florence, 50019, Sesto Fiorentino, FI, Italy

e-mail: francesca.torrini@lens.unifi.it

Integrated microfluidic lab-on-chip devices, combined with 3D printing and photonic technologies, hold significant potential for advancing the study of biophysical and biomedical processes. In this framework, we developed a modular LoC device that integrates two functional units: one for live-cell imaging and one for downstream, label-free optical sensing, enabling in-line analysis of both intracellular processes and extracellular secretome signatures.

The chip prototype is fabricated using stereolithography and a molding replication process based on perfluoropolyether, a UV-curable polymer with excellent anti-fouling properties, which minimize non-specific biomolecule adsorption [1,2]. By combining these two functionalities on a single chip, we designed a novel device to investigate the brain-periphery pathway using a simplified model. As a case study, we investigated the amyloidogenic cleavage of the amyloid precursor protein (APP) by the β-secretase Bace1, a well-characterized molecular event involved in the early stages of Alzheimer’s disease (AD), which leads to the release of soluble APP β (sAPPβ). APP processing dynamics and particularly the secretion of sAPPβ, recently re-evaluated as potential AD early biomarker in biofluids [3], were investigated in neuronal cell models (SH-S5Y). Specifically, cells were transfected with APP fused to monomeric fluorescent proteins at both termini (mChAPPmGFP), allowing the visualization of proteolytic cleavage events through changes in the blue/green fluorescent ratio [4]. To mimic a neuroinflammation state and investigate biochemical and bioanalytical changes relative to a healthy baseline, cells were co-transfected with Bace1 to enhance APP cleavage.

Three compartmentalized cell culture chambers are used for cell confinement and phenotyping under physiological conditions. The integration of a valve enables fluidic connection with the downstream detection module for biomolecule tracking via a trapping surface functionalized with bioreceptors for specific analyte detection. The opening of the valve is designed to simulate the passage of biomolecules across a damaged or breakdown blood-brain barrier, as occurs during neuroinflammatory conditions. Fluorescence imaging was first performed off-chip to validate the construct, followed by on-chip integration. sAPPβ levels in the culture medium were initially quantified using a flow cytometer bead-based immunoassay. In parallel, an affinity-based detection strategy was developed using surface plasmon resonance system and a portable interferometric sensor to characterize binding kinetic/affinity parameters and analytical figures of merit (selectivity, repeatability, limit of detection and quantification). The portable sensing module was then connected in-line with the microfluidic chip, enabling real-time correlation between intracellular dynamics and secretome analysis. This integrated platform holds significant potential for monitoring the effects of disease-modifying therapies and for studying the diagnostic potential of biomarkers reflecting the brain status in peripheral fluids [4], with important implications for non-invasive diagnosis of neuropathologies, especially in the era in which in vitro models are increasingly approaching clinical relevance.

REFERENCES

[1] C. Credi, M. Levi et al., Appl. Surf. Sci. 404, 268-275 (2017).

[2] C. Credi, C. Dallari et al., Bioengineering 10 (6), 676 (2023).

[3] C. Capitini, A. Bigi et al., iScience 26, 106611 (2023).

[4] F. Torrini, M. Gil-Garcia et al., Trends Pharmacol. Sci. 46, 468-479 (2025).