Mechanisms and time scales of free-charge generation in organic photovoltaics: hot and fast or cold and slow

Veljko Janković1,2 and  Nenad Vukmirović2

1Institute of Physics, Faculty of Mathematics and Physics, Charles University in Prague, Ke Karlovu 5, 121 16 Prague 2, Czech Republic

2Scientific Computing Laboratory, Center for the Study of Complex Systems, Institute of Physics Belgrade, University of Belgrade,

Pregrevica 118, 11080 Belgrade, Serbia

e-mail:veljko.jankovic@ipb.ac.rs

The physical mechanisms and time scales of light-to-charge conversion in photoexcited donor/acceptor organic solar cells have recently been heavily debated. The interpretation of experimental signals stemming from ultrafast spectroscopic experiments suggests that free-charge generation mainly occurs on a subpicosecond time scale following the excitation by virtue of high-energy (“hot”) delocalized interfacial charge transfer (CT) states [1]. On the other hand, there is experimental evidence that free carriers are predominantly generated on time scales ranging from tens to hundreds of picoseconds out of the lowest-energy (“cold”) CT state, which is strongly bound and localized [2,3].

We study charge separation in a one-dimensional model of an interface between two organic semiconductors, both on ultrashort and on much longer time scales. We obtain that free charges present on a subpicosecond time scale following the photoexcitation are mainly directly optically generated from the ground state thanks to the resonant mixing between states of donor excitons and free charges [4]. However, on the same time scale, we find that the majority of photogenerated charges still remain bound in form of donor or CT excitons [5]. We obtain that their further separation on longer time scales is weakly electric field- and temperature-dependent and is enabled by the synergy between carrier delocalization and moderate disorder [6].

REFERENCES

[1] G. Grancini et al., Nat. Mater. 12, 29 (2013).

[2] K. Vandewal et al., Nat. Mater. 13, 63 (2014).

[3] A. Grupp et al., J. Phys. Chem. Lett. 8, 4858 (2017).

[4] V. Janković and N. Vukmirović, Phys. Rev. B 95, 075308 (2017).

[5] V. Janković and N. Vukmirović, J. Phys. Chem. C 121, 19602 (2017).

[6] V. Janković and N. Vukmirović, J. Phys. Chem. C 122, 10343 (2018).