- Anisotropic nanostructures for broadband achromatic polarization-insensitive meta-lenses

Zameer Bharwani1,2, Wei-Ting Chen1, Alexander Y. Zhu1, Jared Sisler1,2 and Federico Capasso1

1University of Waterloo, Waterloo, ON, N2L 3G1, Canada

2Harvard John A. Paulson School of Engineering and Applied Sciences, Harvard University, Cambridge, MA, 02138, USA

e-mail: weitingchen@seas.harvard.edu, capasso@seas.harvard.edu

Metasurfaces have attracted widespread attention due to an increasing demand of compact and wearable optical devices [1-7]. For many applications, polarization-insensitive metasurfaces are highly desirable, and appear to limit the choice of their constituent elements to isotropic nanostructures [8]. This greatly restricts the number of geometric parameters available in design. Here, we demonstrate a polarization-insensitive metalens using otherwise anisotropic nanofins which offer additional control over the dispersion and phase of the output light. As a result, we can render a metalens achromatic and polarization-insensitive across nearly the entire visible spectrum from wavelength λ = 460 nm to 700 nm, while maintaining diffraction-limited performance. The meta-lens is comprised of just a single layer of TiO2 nanofins and has a numerical aperture of 0.2 with a diameter of 26.4 µm. The generality of our polarization-insensitive design allows it to be implemented in a plethora of other metasurface devices with applications ranging from imaging to virtual/augmented reality. In addition, we also explore and present an alternative approach to design and select nanostructures using machine learning which is in contrast to the conventional numerical methods deployed by simulation software such as Lumerical.

REFERENCES

[1] S. Wang et al., Nat. Com. 8, 187 (2017).

[2] X. Zhang et al., Nanoscale 10, 9304 (2018).

[3] D. Lin et al., Opt. Exp. 26, 24835 (2018).

[4] G. Yoon et al., Sci. Rep 8, 9468 (2018).

[5] E. Arbabi et al., Optica 3, 628 (2016).

[6] E. Arbabi et al., Optica 4, 625 (2017).

[7] S. Shrestha et al., Light Sci. Appl. 7, 85 (2018).

[8] M. Khorasaninejad et al., Nano Lett 17, 1819 (2017).