Inverse Design for Femtosecond-Laser Photonic Surfaces with Direct Gradient Optimization

A. Haboub1, A. Khelif 2, and B. Aissa1

1Qatar Environment and Energy Research Institute, Materials Center,

Hamad Bin Khalifa University, Doha, Qatar.

2College of Science and Engineering, Hamad Bin Khalifa University, Doha, Qatar.

e-mail: amhaboub@hbku.edu.qa

The inverse design of photonic surfaces produced by high-throughput femtosecond laser processing is limited by a strongly non-linear, many-to-one mapping from laser parameters (power, speed, hatch spacing) to the resulting optical spectrum. Tandem Neural Networks (TNNs) mitigate this ill-posedness by pairing a forward surrogate with a separately trained inverse network, but they still rely on artificial noise injection to uncover diverse solutions and still explore the design space only sparsely. We propose Direct Gradient Optimization (DGO), a single-network alternative that treats the pre-trained forward surrogate as a differentiable proxy for the laser–material interaction and back-propagates errors all the way to the process parameters. This strategy (i) eliminates inverse-model training, (ii) allows the embedding of practical constraints such as laser-power penalties to be encoded directly in the loss function, and (iii) yields transparent sensitivity information through native gradients and SHapley Additive exPlanations (SHAP).

Two optimization modes are assessed: a single-start DGO and a parallel multi-start “Tournament” DGO that launches multiple random seeds, runs a brief qualification phase, and refines only the five most promising candidates. Across 10000+ inverse-design tasks, Tournament-DGO cuts the average spectral root-mean-squared error (RMSE) from 1.29 % (best TNN) to 0.70 %, and boosts design novelty (quantified by the Normalized Euclidean Parameter Distance, NEPD) from 0.26 to 0.38. A SHAP-based meta-analysis shows that convergence is dominated by the initial hatch spacing, explaining why broad sampling followed by pruning addresses this sensitivity. DGO delivers state-of-the-art accuracy, enhanced robustness, and diverse inverse solution without noise heuristics or extra model training. Its ability to embed manufacturability constraints directly in the optimization loop establishes a powerful, energy-aware platform for scalable photonic devices, thermal emitters, and meta surfaces.

Figure 1. Comparison of TNN (left) and DGO (right) across 10,000+ inverse-design tasks. Points show spectral RMSE vs design novelty (NEPD). DGO achieves lower mean error (0.70\% vs 1.29\%) and higher novelty (0.38 vs 0.26).

REFERENCES

[1] M. Park, L. Grbčić, et al., Adv. Sci. 11(26), 2401951 (2024).

[2] L. Grbčić, M. Park, et al., npj Comput. Mater. 11(1), 35 (2025).