Raman spectroscopy as a predictive tool for Laser-Induced Graphene from wooden biomass

J. Nyga1, K. Słowiński1, A. Władziński 1, M. Szczerska 1, J. Bąbińska2, A. Dąbrowska2, M. Babińska 1

1Department of Metrology and Optoelectronics, Faculty of Electronics, Telecommunications and Informatics, Gdańsk University of Technology, Gdańsk, Poland

2Department of Building Engineering, Faculty of Civil and Environmental Engineering, Gdańsk University of Technology, Gdańsk, Poland

e-mail: s188631@student.pg.edu.pl

The synthesis of laser-induced graphene (LIG) is a relatively new and rapidly developing field of materials engineering, offering new opportunities for fabricating functional carbon structures for optoelectronic and sensing applications [1, 2]. However, the wooden materials, despite their promising properties, remain poorly understood in the context of LIG formation [3].

In this work, the use of Raman spectroscopy as a predictive, pre-LIG diagnostic tool to assess wood as a precursor. Presented data obtained from wood samples - spruce, in raw form as offcuts. Raman spectra were collected using an 830 nm laser (near infrared - to reduce background fluorescence) [4], revealing key fingerprints related to cellulose/lignin ratio and sample unity. These factors appear to influence the result of LIG structure - morphological uniformity, low defect density, and potential electronic conductivity relevant for sensing applications.

Preliminary results of spectroscopy show high potential of material carbonization [3]. A relatively simple workflow is proposed, using baseline correction, and highlight peaks, to pre-select wood and extract well-structured graphene-like carbon parts, as illustrated in Figure 1. Raman spectroscopy appears to be a promising predictive tool to suggest which wooden materials can form into effective bio-microelectrodes with potential utility in optical and electrochemical sensing fields.

Figure 1. Raman spectroscopy for wooden material classification.

This research was supported by the "Excellence Initiative – Research University" (IDUB) program: Technetium Talent Management Grants (7/1/2024/IDUB/III.4c/Tc), by the “European Cooperation in Science and Technology” (COST Action CA21159), and by the Gdańsk University of Technology, Faculty of Civil and Environmental Engineering Project Fund (NR038216).

REFERENCES
[1] S. K. Lengger et al., “Laser-induced graphene formation on different wood species: Dependence of electronic performance on intrinsic features of certain types of wood”, 
doi: 10.1016/j.susmat.2024.e00936.
[2] K. Avinash and F. Patolsky, “Laser‑induced graphene structures: From synthesis and applications to future prospects”, doi: 10.1016/j.mattod.2023.10.009.
[3] R. Ye et al., “Laser‑induced graphene formation on wood”, doi: 10.1002/adma.201702211.
[4] A. Władziński et al., “Biomarker detection in the wastewater phantom”, doi: 10.1002/jbio.202500003.