Ultrafast All-Optical Switching due to Photon-Avalanche-Like Effects in Quantum Wells

A. G. Buzykin1 and E. Yu. Perlin1,2

1 St Petersburg Peter the Great Polytechnic University, St Petersburg, Russia

2ITMO University, St. Petersburg, Russia

e-mail:Buzykin7@gmail.com

In this study, several new features of nonlinear optical excitation of materials with quantum wells (QWs) under short laser pulses are treated. The following processes were considered in previous works:

a) “traditional” photon-avalanche (PA)-like processes (type I) in systems of impurity ions [1] and in deep QWs [2]: for this type of processes the photon absorption and the energy transfer between electrons are involved in different elementary events;

b) type II PA-like processes: single- or many-photon [3] absorption and the energy transfer are involved in one elementary event in bulk crystals with special types of the energy band structure [4], wide-gap crystals with high concentration of deep impurities [5], and QWs [6];

c) efficient up-conversion and light-with-light controlling due to both type I and type II PA processes in deep QWs [6].

Quantum-mechanical calculations of the probabilities of elementary processes involved in phenomena under consideration were performed within second or higher-order perturbation theory. The results of these calculations were applied to analyze kinetics of non-equilibrium electron-hole pairs producing, optical transmission, and optical switching the media between states with essentially different optical and/or electrical properties.

General nonlinear oscillation analysis is applied where laser radiation intensity j is considered as a bifurcation parameter [7] In general, theoretical description of type II PA dependence from bifurcation parameter value is observed.

It was shown that in all cases under consideration a rather narrow region of laser intensities j appears where both populations of electron states and optical transmission dramatically change even at small change of j. A number of crystals and heterostructures, whose electron band structure and geometric parameters allow the above-described transient nonlinear processes of photoexcitation and optical switching, are described in detail.

Typical times τ and densities of light energy E for all-optical switching are estimated as τ~0.1÷10 ns and E~0.1÷10 pJ/cm2 respectively.

REFERENCES

[1] S. Guy, M. F. Joubert, B. Jacquier, Phys. Rev. B 55, 8240 (1997).

[2] E. Yu. Perlin, Journal of Luminescence 94-95, 249 (2001).

[3] E.Yu. Perlin, A.V. Ivanov, R.S. Levitskiî, JETP, 101, 357 (2005).

[4] A.V. Ivanov, A.A. Popov, E.Yu. Perlin, J. Opt. Technol. 85, 599 (2018).

[5] A.A. Popov, A.V. Ivanov, E.Yu. Perlin, Opt. Spectrosc., 125, 213 (2018).

[6] E.Yu. Perlin, M.A. Bondarev, A.V. Ivanov, A.A. Popov, Opt. Spectrosc., 125, 1006 (2018).

[7] N.N. Bautin, Mat. Sbornik N.S. 30(72), 181 (1952) (in Russian).